Question 1:

Based on the videos, fill in Part 1 and Part 2 for the black sensor and part 3 based on the video.

Part 1: Sensor Type

- Supply power to your sensor; brown is 12V, blue is 0V and black is output.

- Approach the sensor with the different materials provided by your lab supervisor. Observe the LED on the sensor and determine which of the materials turned the sensor.

Metal, magnet, plastic

Part 2: Sensor Output Circuit Type Determination (PNP/NPN)

- Supply power to your sensor; brown is 12V, blue is 0V and black is output.

- Measure the output voltage once without an object nearby and once with a detectable object.

Vout (without the presence on an object) = --------------------

Vout (with the presence on an object) = --------------------

- The sensor output circuit is: ---------------------------

Part 3: Hysteresis Phenomenon

- Connect the proper circuit to your sensor type. Choose between Figure 1 and Figure 2.

- Approach the sensor with a proper material. Note the turn on and off of the LED you connected.

- Measure and record the turn on/off distance and record them.

Turn on distance = ----------------------------- (mm)

Turn off distance = ----------------------------- (mm)

Figure 1		              		Figure 2

Question 2:

Watch the video name "Relay", how did we test the normally open contact? Explain.

Question 3:

Based on the knowledge you obtained from the video "part 3", connect the circuit shown in Figure 2 above on the following breadboard.